
Emergency Number: (517)496-3780PRIVATE 

______________________________________________________________________

Section 1  


Product Name and Information
______________________________________________________________________

Product(Trade Name and Synonyms):  Frick NG 3
Chemical Name: Polyalkylene Glycol

Chemical Family: Polyglycol

Formula: Proprietary

CAS#: Proprietary

______________________________________________________________________

Section 2


Components and Hazard Statement
______________________________________________________________________

This product is non-hazardous. The product contains no known carcinogens. No 

special warning labels are required under OSHA 29 CFR 1910.1200.

______________________________________________________________________

Section 3


Safe Handling and Storage
______________________________________________________________________

Handling. Do not take internally. Avoid contact with skin, eyes, and clothing. Upon contact with skin, wash with soap and water. Flush eyes with water for 15 minutes and consult physician. Wash contaminated clothing before reuse. 

Storage. Keep container tightly sealed when not in use.  


page 1

______________________________________________________________________

Section 4


Physical Data

______________________________________________________________________

Appearance: Clear liquid; gray to brown tint

Boiling Point: >300°F

Vapor Pressure: <0.1 mmHg @ 20°C

Specific Gravity(water=1):1.04

Volatiles, Percent by Volume: 0%

Odor: slight 

Solubility in Water: soluble

Evaporation Rate(butyl acetate=1):nil

______________________________________________________________________

Section 5


Fire and Explosion Hazards

______________________________________________________________________
Flash Point (by Cleveland Open Cup):204°C(400°F)

Flammable Limits: not established

Autoignition Temperature: no data

HMIS Ratings:


Health:

0



Flammability:    
1


Reactivity:      
0

NFPA Ratings: not established

Extinguishing Media: Water spray; Dry chemical; CO2 foam

Unusual Fire and Explosion Hazards: None

Special Fire Fighting Techniques: Burning fluid may evolve irritating/noxious fumes.      Firefighters should use NIOSH/MNSA-approved self-contained breathing apparatus.

Do not spray water directly on fire. This may cause splashing and frothing of hot liquid.


page 2

______________________________________________________________________

Section 6


Reactivity Data
______________________________________________________________________

Stability: Stable
Hazardous Polymerization: Will not occur

Incompatible Materials: Strong oxidizers and materials incompatible with alcohols

Conditions to Avoid: Excessive heat

Hazardous Decomposition Products: Analogous compounds evolve carbon monoxide,          carbon dioxide, and other unidentified fragments when burned. See Section 5.

______________________________________________________________________

Section 7


Health Hazard Data
______________________________________________________________________

Threshold Limit Value: not established

Situations to Avoid: Avoid breathing oil mists

First Aid Procedures: 


Ingestion: DO NOT INDUCE VOMITING. Consult physician at once. DO NOT 

    give anything by mouth if the person is unconscious



    or having convulsions. 


Inhalation: Product is not toxic by inhalation. If oil mist is inhaled, remove to 

     fresh air and consult physician.


Contact:   Wash exposed skin with soap and water. Flush eyes with large                                       quantities of water and consult physician immediately.

To the best of our knowledge, the toxicological properties of these compounds have not been fully investigated. Analogous compounds are considered to be essentially non-toxic.

______________________________________________________________________

Section 8


Personal Protection Information
______________________________________________________________________

Respiratory Protection: Use in well ventilated area

Ventilation: Mechanical exhaust needed at elevated temperatures

Protective Gloves: Not required, but recommended, especially for long term exposure

Eye/Face Protection: Goggles


page 3

______________________________________________________________________

Section 9


Spill or Leak Procedures
______________________________________________________________________

In Case of Spill: Wear suitable protective equipment, especially goggles. Stop source of spill. Dike spill area. Use absorbent materials to soak up fluid(i.e. sand, sawdust, and commercially available materials). Wash spill area with large amounts of water. Properly dispose of all materials.

______________________________________________________________________

Section 10


Waste Disposal Methods
______________________________________________________________________

Incinerate this product and all associated wastes in a licensed facility in accordance with Federal, state, and local regulations.

The information in this material safety data sheet should be provided to all who use, handle, store, transport, or are otherwise exposed to this product. We believe the information in this document to be reliable and up to date as of the date of publication, but makes no guarantee that it is. 

6/99


page 4

