CPI Engineering Services, Inc.






Frick Company

2300 James Savage Rd.






100 CV Ave.

Midland, MI  48642







Waynesboro, PA  17268


Emergency Number: (517) 496-3780
Section 1  


Product Name and Information

Product(Trade Name and Synonyms):  FRICK #3

Chemical Family: Petroleum Hydrocarbons

Formula: Proprietary





CAS#: Proprietary

Section 2


Components and Hazard Statement

This product is non-hazardous. The product contains no known carcinogens. No 

special warning labels are required under OSHA 29 CFR 1910.1200.

Section 3


Safe Handling and Storage

Handling. Do not take internally. Avoid contact with skin, eyes, and clothing. Upon contact with skin, wash with soap and water. Flush eyes with water for 15 minutes and consult physician. Wash contaminated clothing before reuse. 

Storage. Keep container tightly sealed when not in use. 







Page 1

Section 4


Physical Data

Appearance: Colorless liquid


Vapor Pressure: nil

Boiling Point: about 650°F at 760 mm Hg.
Specific Gravity(water=1):
0.879

Volatiles, Percent by Volume: Nil


Odor: mild

Evaporation Rate(butyl acetate=1): nil

Solubility in Water: insoluble

Section 5


Fire and Explosion Hazards

Flash Point (by Cleveland Open Cup): 250°C (485°F)

Flammable Limits: not established

Autoignition Temperature: no data

HMIS Ratings:

Health:

0


Flammability:    
1

Reactivity:      
0

NFPA Ratings: not established

Extinguishing Media:  Dry chemical; CO2 foam

Fire and Explosion Hazards: Class IIIB Combustible Liquid (NFPA)

Special Fire Fighting Techniques: Special Fire Fighting Procedure:  Water or foam may cause frothing.  Use water to keep fire-exposed containers cool.  Water spray may be used to flush spills away from exposures.

Section 5


Fire and Explosion Hazards – (continued)

Unusual Fire and Explosion Hazards:  Products of combustion may contain carbon monoxide, carbon dioxide and other toxic materials.  Do not enter enclosed or confined space without proper protective equipment including respiratory protection.







Page 2

Section 6


Reactivity Data

Stability: Stable
Hazardous Polymerization: Will not occur

Incompatible Materials: Strong Oxidizers

Conditions to Avoid: Excessive heat, sparks and flames.

Hazardous Decomposition Products: Analogous compounds evolve carbon monoxide, carbon dioxide, and other unidentified fragments when burned. See Section 5.

Section 7


Health Hazard Data

Threshold Limit Value: 5mg/m3 for oil mist

Situations to Avoid: Avoid breathing oil mists

First Aid Procedures: 

Ingestion: DO NOT INDUCE VOMITING. Consult physician at once. DO NOT 

    give anything by mouth if the person is unconscious

    or having convulsions. 

Inhalation: Product is not toxic by inhalation. If oil mist is inhaled, remove to 

     fresh air and consult physician.

Contact:   Prolonged exposure may irritate the skin. Wash exposed skin with       

    soap and water.

To the best of our knowledge, the toxicological properties of these compounds have not been fully investigated. Analogous compounds are considered to be essentially non-toxic.







Page 3

Section 8


Personal Protection Information

Respiratory Protection: Use in well ventilated area

Ventilation: Local exhaust

Protective Gloves: Should be worn when the potential exists for prolonged or repeated skin contact.  NBR or Neoprene recommended.

Eye/Face Protection: Goggles, safety glasses with side shields.

Section 9


Spill or Leak Procedures

In Case of Spill: Wear suitable protective equipment, especially goggles. Stop source of spill. Dike spill area. Use absorbent materials to soak up fluid(i.e. sand, sawdust, or commercially available materials). Wash spill area with large amounts of water. Properly dispose of all materials.

Section 10


Waste Disposal Methods

Incinerate this product and all associated wastes in a licensed facility in accordance with Federal, state, and local regulations.

The information in this material safety data sheet should be provided to all who use, handle, store, transport, or are otherwise exposed to this product. CPI believes the information in this document to be reliable and up to date as of the date of publication, but makes no guarantee that it is. 







Page 4

1/2000

