Model:
 CARRIER 19XL, XR, XRV CENTRIFUGAL CHILLER





Frequency:

Task and Actions

Year 1
 
 
 
 
 
ROUTINE OPERATING INSPECTION

3    
     
     
     
     
     


REPORT TO CUSTOMER UPON ARRIVAL

x    
     
     
     
     
     


CHECK GENERAL MACHINE OPERATION

x    
     
     
     
     
     


CHECK CONTROL, POWER AND PIPING

x    
     
     
     
     
     


CHECK SAFETY / OPERATING CONTROLS

x    
     
     
     
     
     


CHECK GAUGES / INDICATOR LIGHTS

x    
     
     
     
     
     


LOG CHW/BRINE IN TEMPERATURE

x    
     
     
     
     
     


LOG CHW/BRINE OUT TEMPERATURE

x    
     
     
     
     
     


LOG CHW/BRINE FLOW DELTA P

x    
     
     
     
     
     


LOG CHW/BRINE PUMP DELTA P

x    
     
     
     
     
     


LOG COOLER REFRIGERANT TEMPERATURE

x    
     
     
     
     
     


LOG COOLER PRESSURE

x    
     
     
     
     
     


LOG COOLER LEAVING TEMPERATURE DIFFERENCE
x    
     
     
     
     
     


LOG COND WATER IN TEMPERATURE

x    
     
     
     
     
     


LOG COND WATER OUT TEMPERATURE

x    
     
     
     
     
     


LOG COND WATER FLOW DELTA P

x    
     
     
     
     
     


LOG COND WATER PUMP DELTA P

x    
     
     
     
     
     


LOG CONDENSER REFRIGERANT TEMP

x    
     
     
     
     
     


LOG CONDENSING PRESSURE

x    
     
     
     
     
     


LOG COND SUBCOOLER TEMPERATURE

x    
     
     
     
     
     


LOG COND LEAVING TEMP DIFFERENCE

x    
     
     
     
     
     


LOG VFD OPERATION AS APPLICABLE

x    
     
     
     
     
     


LOG OIL LEVEL

x    
     
     
     
     
     


LOG OIL SUMP PRESSURE

x    
     
     
     
     
     


LOG OIL SUMP TEMPERATURE

x    
     
     
     
     
     


LOG BEARING TEMPERATURE

x    
     
     
     
     
     


LOG OIL SUPPLY PRESSURE

x    
     
     
     
     
     


LOG REFRIGERANT DRY-EYE INDICATOR STATUS
x    
     
     
     
     
     


CHECK APPROACHES THROUGH CALCULATION

x    
     
     
     
     
     


CLEAN UP WORK STATION

x    
     
     
     
     
     


REPORT TO CUST, ADVISE & OBTAIN SIGNATURE
x    
     
     
     
     
     

Model:
 CARRIER 19XR, XL CENTRIFUGAL CHILLER

                                       Frequency:

Task and Actions

Year 1
 
 
 
 
 
ANNUAL PREVENTIVE MAINTENANCE

1    
     
     
     
     
     


REPORT TO CUSTOMER UPON ARRIVAL

x    
     
     
     
     
     


LEAK CHECK WITH LEAK DETECTOR

x    
     
     
     
     
     


CALIBRATE FLOW SWITCHES

x    
     
     
     
     
     


CALIBRATE MOTOR AMPA

x    
     
     
     
     
     


CALIBRATE VOLTS

x    
     
     
     
     
     


SECURE CIRCUITS, LOCKOUT AND TAG

x    
     
     
     
     
     


REVIEW AND EVALUATE LOG READINGS

x    
     
     
     
     
     


PULL OIL SAMPLE FOR ANALYSIS

x    
     
     
     
     
     


REPLACE OIL FILTER(S) (A/R)

x    
     
     
     
     
     


DRAIN WATER FROM CONDENSER WATER BOX

x    
     
     
     
     
     


REMOVE CONDENSER HEAD

x    
     
     
     
     
     


CLEAN/BRUSH CONDENSER TUBES

x    
     
     
     
     
     


(RE) ASSEMBLE CONDENSER HEAD

x    
     
     
     
     
     


CHECK GAUGES / INDICATOR LIGHTS

x    
     
     
     
     
     


REPLACE OIL RECLAIM FILTER & STRAINER

x    
     
     
     
     
     


MEGGER COMPRESSOR MOTOR < 480V

x    
     
     
     
     
     


RESTORE POWER

x    
     
     
     
     
     


LEAK CHECK DISASSEMBLED JOINTS

x    
     
     
     
     
     


CLEAN UP WORK STATION

x    
     
     
     
     
     


REPORT TO CUST, ADVISE & OBTAIN SIGNATURE
x    
     
     
     
     
     

