	YORK PROCESS SYSTEMS - DESIGN DRAFTING STANDARD

PACKAGE RIGGING DIAGRAM - EIGHT LUGS
	BA-18004
ISSUE: 1 Feb 96



[image: image1.png]


WARNING:
Unit may be top heavy.  Lifting operators must use extreme care to check the level and stability of the load before lifting the load more than a few inches.  Impose an imbalance by sequentially adding weight to each corner and carefully observing the load reaction to make sure the load does not shift.  Balancing chains, cables or straps are essential in both directions to prevent load shift and instability during rigging.  Call YORK Process Systems Engineering for an estimate of the location of the center of gravity of the package if one is not given.  The center of gravity may not be located in the center of the package.

NOTES:

1) Reference OSHA Safety And Health Standards (29 CFR 1910), sections 1910.179 and 1910.184.

2) Hooks, chains, cables and spreader bars shall meet manufacturer's recommendations and shall not be overloaded.

3) This unit shall be lifted using the eight lifting lugs welded to the base as shown above.  Shackles and screw pins shall be provided (by others) as shown in Figure 2 as minimum.

4) Spreader bars and balancing chains must be used to prevent instability and damaging or straining system piping, instrumentation or shells.

5) Adjust cables or chains to insure that the package (skid) is stable and lifted level.

6) Lifting must be done by a qualified operator.
	[image: image2.png]HNCHOR SHACKLE W/ SCREV PIN
HIN RECOHMENDED LDAD
RATING: 24,0008, NOMNAL.

SOLE ST LA




[image: image3.png]1 1

|—— sProor wrs ——|

VBN Ak

L —

SPREADER B4R
B OTHERS

CHIN OR GIBLE
RATED ACCORDNGLY




LIFTING LUG CAPACITY, LB/LUG

	NOMINAL BEAM

MEMBER SIZE
	CABLE ANGLE

	
	VERTICAL
	45(

	W8 X 28
	15,200
	14,990

	W8 X 67
	15,200
	14,990

	W10 X 49
	19,400
	14,990

	W12 X 26
	21,200
	14,990

	W12 X 40
	21,200
	14,990

	W16 X 36
	21,200
	14,990

	W18 X 46
	21,200
	14,990


Figure 2





Figure 1





NOTE: lug design may vary.








	Original:
SDH
Revision:
4
by
HWK
Apprd: 

Rev Date:
11 Jan 06
File: v:\manuals\rel\dds\ba-18004.doc
	REVISION RECORD:  Added NOTE regarding lug design on page 2.
	Page 1 / 2



