Page 1 of 5

[image: image1.wmf]10244 Hedden Road Evansville Indiana 47725

PHONE: (812) 867-4730 FAX: (812) 867-4744

DATE: _10/20/2009____________________________________

FILTER MODEL: _VAG3001SP/1_________________________

CUSTOMER ORDER NUMBER: _ITEM# 560A0057H01________
COMPRESSED AIR, GAS & VACUUM FILTER

Filter Function:

The VAG and VVS filter are designed to remove unwanted particulate matter from air, natural gas and other gases in compressed gas systems, as well as vacuum systems, protecting pumps or other process equipment.

Be certain that all filter components, including vessel, cartridge and gasket are chemically and physically compatible before installation.

Installation:

Piping into which the filter is to be installed must be absolutely clean (free from dirt and grit).  The filter must be mounted vertically (unless otherwise designed for horizontal mount), and as close to the point of usage of the filtrate as possible.

Be sure to install in accordance with the flow arrows.  Flow is directed from the inside of the cartridge to the outside (unless specifically noted by special design). 

The filter is designed for top servicing, therefore be sure to allow sufficient space for serving above the filter.  In the event of situations where single cartridge filters require hanging and bottom servicing is required be sure to allow sufficient space below for servicing.

Install pressure gauges or a differential pressure gauge on filter or system to measure filter pressure drop.

Maintenance Procedure:

Prior to opening the filter be sure to depressurize the system.  (In vacuum systems, bring system up to atmospheric pressure).  Open the drain (and if equipped, the vent), to remove all accumulated particulate that may have been mechanically separated from the sump.  Open the filter closure and remove the filter cartridge.

Install new or cleaned cartridge (cartridges, if more than one).  Check the vessel closure gasket for signs of deterioration and replace if required.  All closures bolts should be well lubricated with and common thread lubricant.

Be sure to align closure gasket and close filter closure.  Torque bolting to obtain seal.

Close vent and drain, pressurize system and check for leaks.

Cleaning Procedure:

Cartridges installed in your VAG and VVS filter are cleanable.  Each time the cartridge is cleaned, the service life will be diminished and is dependant upon the contaminants becoming entrapped within the fabric or felt.  When the service life becomes intolerable, a new filter cartridge should be installed.

Washing:

Washing your cartridge is accomplished by soaking and agitating the cartridge in a suitable solution for the contaminants involved.  Hot water (140-160 F) containing a detergent maintaining a PH of 7 can be used. After cleaning, the cartridge should be rinsed well and completely air dried before placing back into service.

Always keep a spare cartridge to prevent any downtime during maintenance cycles.

Operating Instructions:
Since the filters function is to remove solids from compressed air, gas or vacuum systems, a pressure drop across the filter will rise as the unwanted contaminants are retained by the filter cartridge.

The life expectancy of a cartridge will vary upon the amount of solids in the air or gas stream; therefore, it is recommended that the pressure drop of the filter be monitored frequently to determine its life.

Pressure taps on the nozzles are offered as an accessory where pressure gauges may be attached.  If the filter is not equipped with pressure taps, monitor the pressure differential across the filter as closest to the inlet ad possible.

The filter should be serviced and maintained at a 10 PSI maximum pressure differential.

The cartridges are cleanable and may be hand washed, with the exception of some unwashable media.  If the cartridges show signs of wear, they should be replaced.

By – Pass Guidelines (For duplex units only)

Valves should be opened and closed slowly.  Deviation from the procedures listed could cause the system to shutdown.

Switching service from filter #1 to filter #2

Open valve #2, Open valve #4, Close valve #3, Close Valve #1

Switching service from filter #2 to filter #1

Open valve #1, Open valve #3, Close valve #4, Close Valve #2

[image: image2.wmf]PARTS LIST

Element Number: _S397-8SS_________           QTY: _1_

Gasket Number: __BFG04____________
    QTY: _1_

� EMBED AutoCAD.Drawing.14  ���








VAG – VVS Installation, Operation, and Service Notes


[image: image3.png]minDUSI'l"iaL FiLTer
ManurFacrTurers



_1028438507.dwg

